Leveraging the orthogonality of sinusoidal waves, the encoded signals can be extracted by isolating the transmitted signal (VT) at the desired encoding frequencies (f1,f2, etc.) using Euler’s formula.

Vo1t=VTt.e-i2πf1t,  						(3)

Vo2t=VTt.e-i2πf2t,

Vo3t=VTt.e-i2πf3t, etc.

Lastly, the original signal (Vs) can be demodulated from the extracted signals (Vo) by averaging the signal over a sufficiently long span of time to remove the carrier frequency. We implemented this using a 5th order Butterworth low-pass filter with a cut-off frequency of 3 Hz.